AMERICAN INTERNATIONAL

UNIVERSITY-BANGLADESH

Project Title: Vehicle Safety System

Course Title: Advanced Operating System       Section: [A]

Course Teacher: Md. Arifur Rahman 	         	 Semester: Fall 2021-2022

Date of Submission: 12-12-2021

Group Member Details:

| NO. | Name | ID |
| --- | --- | --- |
| 1 | Fahad Mollah | 18-36702-1 |
| 2 | Fahim Hossain | 18-36954-1 |
| 3 | Ahnaf Sayed | 18-36920-1 |
| 4 | Md Sifat Rahman | 18-36705-1 |

# Table of Contents

| Contents | Page No. |
| --- | --- |
| Project Idea and Problem Statement | 03 |
| Component List | 03 |
| Project Implementation Description | 03 |
| Circuit Diagram | 06 |
| Implemented Code | 07 |
| Reference | 11 |

# Project Idea and Problem Statement

## Idea: Vehicle Safety System

We will create a vehicle safety automation system, when the driver enters and starts the car breath analyzer will check if he is drunk or not then using BP analysis meter and oximeter by checking driver's pulse and oxygen saturation. If the system finds any problem in data it will not start the vehicle and if the system doesn't find any problem in data from the breath analyzer, BP sensor and pulse oximeter it will start the vehicle. There will be a SOS message system if any accident occurs, ambulance and police immediately to place where accident occurred using GPS tracking. Also, there will an eye blink analyzer to detect if he is alive, if there is no eye blinking for 5 or 7 second it will stop the vehicle immediately.

# Component List

| Component | Amount | Price |
| --- | --- | --- |
| Arduino Mega | 1 | 1150 |
| L298N Motor Driver | 1 | 150 |
| Gear Motor | 1 | 200 |
| Buck Converter | 1 | 60 |
| Battery | 1 | 1200 |
| Wires | 1 | 60 |
| MQ 3 Alcohol sensor | 1 | 100 |
| Pulse Sensor | 1 | 300 |
| GPS Module | 1 | 850 |
| GSM Module | 1 | 800 |
| Display | 1 | 300 |

# Project Implementation Description

At first the alcohol sensor will be used for taking persons breath and pulse sensor will collect the bpm and then if any abnormal bpm or alcohol analog value is detected the car will not start. SMS Will be sent. For the device we have use 11.1-volt battery and buck converter for convert 11 volts to 5 volts. As all of the sensor requires 5v. For the eye blinking we have used the open CV library which allows multiple libraries to detect several faces and from that faces we detect eyes. Then the trained data can detect close eyes and open eyes. So, counting time we can detect the eyes are close or open. Time set for the whole detected and that’s detects the blinking.

Figure: Device Implementation

Figure: Device Implementation

Figure: open Cv face detection

Figure: open Cv face detection

Figure: SMS from the GSM Module

# Diagram

## Circuit Diagram

Figure: Circuit Diagram

Figure: Diagram of the project

# Result Analysis

Figure: Heart BPM in different Activities to measure the abnormality

# Implemented Code

#define USE_ARDUINO_INTERRUPTS true    // Set-up low-level interrupts for most acurate BPM math.
#include <PulseSensorPlayground.h>     // Includes the PulseSensorPlayground Library.  
#include <LiquidCrystal_I2C.h>

#include <SoftwareSerial.h> //library for the gsm module
SoftwareSerial mySerial(9, 10);
char msg;
char call;

LiquidCrystal_I2C lcd(0x27,20,4);  //adress of the display

//  Variables
const int PulseWire = 0;       // PulseSensor PURPLE WIRE connected to ANALOG PIN 0         // The on-board Arduino LED, close to PIN 13.
int Threshold = 550;           // Determine which Signal to "count as a beat" and which to ignore.
#define analog_pin 1
float sensor_value;  //variable to store sensor value

#define notDrunk 500   // Define max value that we consider not Drunk
#define Drunk 440   // Define min value that we consider drunk

int motor1pin1 = 2;  // motor driver pin1
int motor1pin2 = 3;  //Motor driver pin 2


                               
PulseSensorPlayground pulseSensor;  


void setup() {  

  Serial.begin(115200);    
    mySerial.begin(9600);// For Serial Monitor
  Serial.println("MQ3 warming up!");
  delay(6000);
  pinMode(motor1pin1, OUTPUT);
  pinMode(motor1pin2, OUTPUT);
  lcd.init();                      // initialize the lcd
  // Print a message to the LCD.
  lcd.backlight();
  // Configure the PulseSensor object, by assigning our variables to it.
  pulseSensor.analogInput(PulseWire);          //auto-magically blink Arduino's LED with heartbeat.
  pulseSensor.setThreshold(Threshold);  

  // Double-check the "pulseSensor" object was created and "began" seeing a signal.
   if (pulseSensor.begin()) {
    Serial.println("We created a pulseSensor Object !");  //This prints one time at Arduino power-up,  or on Arduino reset.  
  }
}



void loop() {

 int myBPM = pulseSensor.getBeatsPerMinute();  // Calls function on our pulseSensor object that returns BPM as an "int".
                                               // "myBPM" hold this BPM value now.

if (pulseSensor.sawStartOfBeat()) {            // Constantly test to see if "a beat happened".
 Serial.println("  A HeartBeat Happened ! "); // If test is "true", print a message "a heartbeat happened".
 Serial.print("BPM: ");                        // Print phrase "BPM: "
 Serial.println(myBPM);  
  lcd.setCursor(3,0);
  lcd.print("BPM:");
  lcd.setCursor(2,1);
  lcd.print(myBPM);// Print the value inside of myBPM.
}

  delay(20);    
 
sensor_value = analogRead(analog_pin);

  Serial.print("Sensor Value: ");
  Serial.print(sensor_value);
 
  if (sensor_value < notDrunk) {
    Serial.println("  |  Status: Not Drunk");
    lcd.setCursor(0,2);
  lcd.print("Status: Not Drunk");
     digitalWrite(motor1pin1, HIGH);
  digitalWrite(motor1pin2, LOW);
  }  else {
    Serial.println("  |  Status: DRUNK");
     lcd.setCursor(0,2);
  lcd.print("Status: DRUNK");
 
     digitalWrite(motor1pin1, LOW);
  digitalWrite(motor1pin2, LOW);
  }
 
  delay(2000);  
 
 
 
 
 
 
 
  // considered best practice in a simple sketch.

}
void SendMessage()
{
  mySerial.println("AT+CMGF=1");    //Sets the GSM Module in Text Mode
  delay(1000);  // Delay of 1000 milli seconds or 1 second
  mySerial.println("AT+CMGS=\"+8801767670530\"\r"); // Replace x with mobile number
  delay(1000);
  mySerial.println("User at risk Latitude(N):23.718580 Longitude(N):90.427551

http://maps.google.com/maps?&z=15@mrt=yp&t=k&q=23718580+90.427551 ");// The SMS text you want to send
  delay(100);
   mySerial.println((char)26);// ASCII code of CTRL+Z
  delay(1000);
}

Eye blinking code

import cv2

# initialize face and eye cascade xml of opencv library to detect face and eyes
face_cascade = cv2.CascadeClassifier(cv2.data.haarcascades + "haarcascade_frontalface_default.xml")
eyes_cascade = cv2.CascadeClassifier(cv2.data.haarcascades + "haarcascade_eye_tree_eyeglasses.xml")

first_read = True

# Video Capturing by opening web-cam
cap = cv2.VideoCapture(0)
# to check for first instance of capturing it will return True and image
ret, image = cap.read()

while ret:
    # this will keep the web-cam running and capturing the image for every loop
    ret, image = cap.read()
    # Convert the recorded image to grayscale
    gray_scale = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)
    # Applying filters to remove impurities
    gray_scale = cv2.bilateralFilter(gray_scale, 5, 1, 1)
    # to detect face and eye
    faces = face_cascade.detectMultiScale(gray_scale, 1.3, 5, minSize=(200, 200))
    if len(faces) > 0:
        for (x, y, w, h) in faces:
            image = cv2.rectangle(image, (x, y), (x + w, y + h), (0, 255, 0), 2)
            # eye_face var will be i/p to eye classifier
            eye_face = gray_scale[y:y + h, x:x + w]
            # image
            eye_face_clr = image[y:y + h, x:x + w]
            # get the eyes
            eyes = eyes_cascade.detectMultiScale(eye_face, 1.3, 5, minSize=(50, 50))
            if len(eyes) >= 2:
                if first_read:
                    cv2.putText(image, "Eye's detected, press s to check blink", (70, 70), cv2.FONT_HERSHEY_SIMPLEX,
                                1, (0, 255, 0), 2)
                else:
                    cv2.putText(image, "Eye's Open", (70, 70), cv2.FONT_HERSHEY_SIMPLEX,
                                1, (255, 255, 255), 2)
            else:
                if first_read:
                    cv2.putText(image, "No Eye's detected", (70, 70), cv2.FONT_HERSHEY_SIMPLEX,
                                1, (255, 255, 255), 2)
                else:
                    cv2.putText(image, "Blink Detected.....!!!!", (70, 70), cv2.FONT_HERSHEY_SIMPLEX,
                                1, (0, 255, 0), 2)
                    cv2.imshow('image',image)
                    cv2.waitKey(1)
                    print("Blink Detected.....!!!!")
                   
    else:
        cv2.putText(image, "No Face Detected.", (70, 70), cv2.FONT_HERSHEY_SIMPLEX,
                    1, (0, 255, 0), 2)
    cv2.imshow('image', image)
    a = cv2.waitKey(1)
    # press q to Quit and S to start
    # ord(ch) returns the ascii of ch
    if a == ord('q'):
        break
    elif a == ord('s'):
        first_read = False
# release the web-cam
cap.release()
# close the window
cv2.destroyAllWindows()

# Reference

- https://ijireeice.com/wp-content/uploads/2019/05/IJIREEICE.2019.7418.pdf

Smart Bus System Using IOT

- https://www.nabto.com/esp8266-for-iot-complete-guide/

ESP8266 for IoT: A Complete Guide

- https://create.arduino.cc/projecthub/ruchir1674/how-to-interface-gps-module-neo-6m-with-arduino-8f90ad

How to Interface GPS Module (NEO-6m) with Arduino

- https://www.seeedstudio.com/blog/2019/11/06/arduino-gps-modules-which-one-to-use-guide-and-comparisons/

Arduino GPS Modules – Which one to use? Comparison and Arduino Tutorial

- https://lastminuteengineers.com/how-rfid-works-rc522-arduino-tutorial/

What is RFID? How It Works? Interface RC522 RFID Module with Arduino

- https://components101.com/articles/buck-converter-basics-working-design-and-operation

Buck Converter: Basics, Working, Design and Operation

- https://www.saftbatteries.com/energizing-iot/types-batteries-iot-devices

Which types of batteries for your IoT devices?

- https://www.cablinginstall.com/standards/network-protocols/article/16465159/cabling-infrastructure-for-the-internet-of-things

Cabling infrastructure for the Internet of Things

- https://www.techshopbd.com/

TechShopBD